LRS-FDQ08 Coaxial Cable Single Channel Video Amplifier 12V

LRS-FDQ08 Video In/Out DC12V Coaxial Cable Video Amplifier

Features:

Material	: Aluminum alloy

Color: Grey + silver

Operating Temperature	10'C~55'C

Operating Humidity	0%~90%RH

Input voltage: DC 12V

Input/output: 75ohms

Input connector: 1 x BNC

Output connector: 1 x BNC

Frequency response: Adjustable

Bandwidth(-3dB): 10Hz to 7.5MHz

Gain: Adjustable, Max. 6dB

Max. output: 2.5Vp-p

Structure: Thin metal casing

Installation: Connect 12V DC to it

Connect input video to "Video in" BNC

Connect output video to "Video out" BNC

After the above operations are completed, you can adjust gain up to output video voltage is 1Vp-p, then adjust HFGAI; You can repeat these operations up to the satisfied result.

Packing List:

1 x Coaxial cable video amplifier

1 x Chinese manual