HD / SD Mobile Digital Car DVB-T TV Receiver Black

DVB-T Twin Tuners HD / SD Mobile Digital Car TV Receiver Box Black

Specification:

RF Input: UHF: 470MHz~862MHz;

VHF:174MHz~230MHz;

Input Impedance:75ohm;

Signal Level: -80dBm~-10dBm;

Connector Type: SMA type terminal x 2;

Channel Band Width: 6MHz, 7MHz, 8MHz;

Constellation: QPSK, 16QAM, 64QAM;

Guard Interval: 1/32, 1/16, 1/8, 1/4;

FEC: 1/2, 2/3, 3/4, 5/6, 7/8;

COFDM Carrier Mode: 2K And 8K Non Hierarchical/Hierarchical;

Transport Stream: MPEG-4AVC/H.264 HP@L4;

MPEG-2 MP@HL/ML;

Video Aspect Ratio: 4:3 / 16:9;

Video System: PAL / NTSC;

Video Output: 1+/-0.3Vpp;

Video Resolution YPbPr Resolution: 720 x 576@25fps/720 x 480@30fps/HD, 1920 x 1080p/1920 x 1080i;

Audio Decoder: MPEG-2/HE_AAC;

Audio Channel: R/L/ST;

Audio Output: 1Vpp 47Kohm Adjustable w/ E Volume;

Supports multiple OSD languages: English, Czech, Danish, French, German, italian, Portuguese, Spanish; RF Input: UHF: 470MHz~862MHz; VHF: 174MHz~230MHz

Main Features: Fully comply with DVB-T Standard Twin tuners with diversity structure TV box for high speed driving Subminiature appearance, Easy to install;

Specially designed for car using;

Supports high speed up to 230km/hours;

Support MPEG-1, MPEG-2, MPEG-4, H.264 decoder (up to 1920x1080P);

Audio Decoder: MPEG-2/HE_AAC(DD/DD +optional);

Support multi-media format : MPG, MKV, AVI, MOV;

Super fast hardware decoding of JPEG picture DVB subtitling ETSI 300473 compliant and OSD teletext;

Supports multiple OSD languages menu, EPG (Electronic Program Guide) with now and next and 7 days events Channel search with automatic/manual scan;

USB2.0 host support upgrade, PVR, time shifting and media play with U-FLASH or HDD RCA audio and video input with all the way ,Three of RCA audio and video output; Standby consumption: less than 0.85W

Packing List

1 x TV receiver

1 x Remote controller (1 x CR2025 button cell)

1 x Antenna

1 x Power cable

1 x Male plug video cable

1 x Remote control receive cable

1 x English user manual