Bracelet User Manual

10086      10086

Foreword

Thank you for using Bracelet--mobile phone partner. It is a healthy ,

Eco-friendly ,high-end ,fashion and wearable product.

This manual in includes : product function ,the way of use and the  operation instruction .Please read it carefully in order to achieve the best using effect and to prevent unnecessary damage .Do not change the accessories of the unit ,disassemble it .otherwise ,it will beyond the range of the warranty.

Package Content : Bracelet ,USB Charging clip ,User manual ,warranty card

Application method

1.Schematic diagram

(According to below diagram , it include A key (up key),B key (down key), OLED display screen ,Connection  identifier ,  charging identifier.speaker.

Microphone.battery holes)

Charing identifier           Microphone

Connection identifier	A key

Screen	                        B key

Speaker

Press the B key for 3 seconds , it will display ”power on” ,again ,it will display ”power off”.

2. Pair and connection

First time use , please connect bracelet with mobile phone by

Bluetooth. Follow the steps below:

Press the B key for 5-6 seconds till the screen display “Pairing”. in this process , it  will show “power on ”first ,don’t let go until it shows“ pairing”)

Turn  on the mobile phone Bluetooth , search device and find “bracelet” ,confirm the connection.

Once connected , there will be a vibration and a little cell phone icon appears on the upper left

Reminding distance depends on the mobile phone brand.

Short press any key to stop the vibration.

3. Answer incoming cells

When the cell incomes, pressing A key or B key to start to talk or directly answer the phone.

When the phone is being dialed out , “ Call” or “Active Call” is displayed ,press the A key or B key to get through the phone cell.(Notice: “ Call ”shows dialing out ,“Active Call” means the call is being connected.

When phone is being answered, short press A key or B key can switch the sound to bracelet and talk by the microphone on bracelet.

When the bracelet is connected, long press A key or B key for 3 seconds to hang up.

4.Transfer to phone

Press the A key for 2 seconds, when active cell, it transfer to the phone;

5.Music player

lf properly connected ,the music playing on the phone can be synchronized.

When music is played, short press A key or B key to make the volume+/-.

If the music should be switch back on the mobile phone ,just turning off media audio connection under the mobile phone Bluetooth option, if this operation is being done on the i phone , this can be done directly over the phone.

times every 3 seconds ,total 9 times ,and try to connect with phone , meanwhile, it shows “Mobile phone without induction”.

It  tries to connect once every 10 minutes until connected or press any key to stop connecting.

When disconnected, press any key will start to search and connect phone device.

6. Reset

If system is halted, such as no display , switch failure, can’t connection etc., turning off or charging and the device will be reset.

7. How to wear it

Put your hands fiat and keep palm down, wear it from hand side (refer to left picture)

Turn to keep it on the best position (refer to right picture)

The max width of the legs are 5CM, please do not open it too wide to break it.

Product Function :

Incoming call reminder with vibrating

Anti-lost reminder with vibrating

Display caller number/ name

Transfer to phone

Answer the incoming call

Synchronize music play

Message notification

OLED screen showing the time

Away from the phone radiation

Alarm clock reminder

Support Chinese traditional; Chinese simplified; English; Korean; Russian; Spanish; French; German language; Italian; Portuguese; Arabic and other languages.

Notice :

1.Please keep the charging clamp and bracelet form an angle of 90 degrees when charging.

2.If it cannot be connected with mobile phone or working properly, please reset or charge it.

3.Charge it timely to avoid power off and clock reset .

4.Maximum distance of two feet is 5cm, do not stretch the feet too far away to prevent fracture.

5.Charging

Charging  more than 2 hours for the first time.

When charging , the screen will display charging logo.

6.Power on and power off

Refer to diagram, up key is A key , down key is B key.

Non intelligent phone of Bluetooth version <2.1 will be required to input password, the password is “0000”.

7.Incoming calls reminder

If  properly connected, bracelet will sustained vibrate when calls coming in.

It  shows caller’s number or name when calls coming in. If you use apple series phone, the caller’s name will be displayed directly on the screen.

Double-click any key to stop the vibration or directly answer the phone.

Press any key for 3 seconds to reject calls.

8.Anti-lost reminder

When the bracelet is 10-20 meters away from the cell phone, bracelet will remind with vibrating, and once every 10 minutes.

When cell is connected, short press A key or B key to make the volume+/-

You can switch the sound between mobile devices and bracelet freely through the Bluetooth setting on mobile.

9.Dial out the last outgoing cell

Double-click A key or B key to Dial out the last outgoing call.

When lt shows “Call”, press any key can start to talk.

10.Hang up phone

Press the B key for 2 seconds, when active cell or outgoing cell, it hang up the phone;

11. Time setting

Press A key for 3 seconds to enter the time adjusting mode.

Pressing B key continuously to set the “hour” time.

Short press A key to enter the minute adjusting mode, press B key to adjust “minute”.

After adjust the minute, short press A key to confirm.

Short press any key, the current time will be displayed.

Short press any function key, the time on the screen will be displayed

lf there is not enough power, Please charge it in time to avoid over discharge, otherwise, the time be disordered.

12. Disconnect and connect

When i watch and mobile phone is disconnected, it will vibrate 3.

13. Safety instruction

Not use any corrosive solvent to clean the product.

Not place it in high temperature circumstance for a long time.

Not soak it in water

Not repair or disassemble it by yourself,

14. Specifications

Model: BRACELET

Battery: Lithium polymer

Bluetooth version: V3.0

Working voltage: 3.3v~4.2v

Working current: 2.0~200 mA

Charging time : 2h

Stand-by time: 48

Bluetooth Bracelet 功能说明

连接标识	充电标识

10086	A键

屏幕	B键

产品规格：

产品型号：蓝牙版本：V3.0

工作电压：3.3V~4.2V

工作电流：45mA 以下

工作频率：2.4GHz

充电时间：2小时

电池类别：聚合物锂离子电池

产品尺寸：内周长18cm

产品重量：40克

待机时间：非连接理论待机（自动进入关机）：1个月以上；连接理论待机：72小时 以上

充电：充电线一端接USB接口可充电设备，另一端充电夹子对准产品底部充电孔并夹紧，OLED 显示屏会出现充电标识。（注：产品内置可充电聚合物锂离子电池，在使用之前先充电3小时以上）

语言版本：支持12种语言：中文繁体；中文简体；英语；韩语；俄语；西班牙语；法语；德语；意大利语；葡萄牙语；阿拉伯语、日语多国语言。

开关机：开关机长按B键6秒为开关机；开机时屏幕显示英文Power on；关机时屏幕显示英文Power off。手机屏幕显出现“确认联机”样.按“是”；BRACELET 显示屏左上角出现耳机样标识，同时产生一次振动提示配对成功。（注：“Pairing”状态下150秒如配对未成功则视为配对失败，需重新配对）

配对：

（1）首次使用要完成手机伴侣与手机的配对；

（2）在关机状态长按 B 键 6 秒，显示屏出现英文“Pairing”进入对码状态；

（3）将手机设置在蓝牙功能开启状态，进入手机蓝牙界面选定“搜索设备”并按“确定”；

（4）手机屏幕出现本产品蓝牙设备“BRACELET”,并确认连接（注：有些手机出现密码输入界面，输入四位数字密码（0000）后按“确定”）；

（5）短按任意功能会唤醒LED屏显示时钟。（注：应及时充电以避免电量耗尽造成时钟重置）

菜单功能：

（1）时钟功能

在开机状态下.按A键3秒进入时间调整状态:显示小时调整，连续按B键调准小时；短按A键显示分钟调整.连续按B键调准分钟；

（2）电话簿功能

在配对成功后，按A键打开菜单，选择电话簿，短按B键，进入电话本同步中且显示同步多少条；完成后会显示同步完成或者同步失败，同步后来电能显示来电者姓名和号码。

（3）语言选择功能

选择语言，短按B键，进入语言选择菜单，按A键选择，再按B键确定。

（4）防丢失功能

当您手机的距离超出10米，BRACELET即有节奏振动提醒您不要遗失手机；

（5）来电提醒，来电显示功能

BRACELET与手机配对成功后在联通状下，BRACELET对手机来电会持续振动提醒；

BRACELET对手机来电会显示来电号码和名字；

直接接听手机或短按任意键可停止振动；若不便接听电话可长按B键切断来电），短按任意键可停止振动响应。（注：在室内或复杂环境下提醒的有效距离会缩短）

(6)通话切换功能

在通话时，双击B键，可以把通话声音从蓝牙转到手机，或者从手机转到蓝牙。

断开与连接：

BRACELET 与手机断连后.会连续振动3组相隔3秒一组，共9次，并主动寻找连接手机；如连接未成功则10分钟内主动连接一次；

BRACELET在断连状态下，按B键为主动搜索连接手机。（注：人体的贴近遮挡有时会产生断连现象）

产品复位：

BRACELET在电池电量低的状态下工作容易易产生死机状态；

如出现屏幕不显示，与手机不连接、振动混乱等死机状态时，充电即可复位。