VC97 3999 LCD Digital Multimeter Auto Range & bag R C F V A meter；

Features:

1.New type holster, streamline design;

2.Large LCD makes the reading clearly;

3.Strong antimagnetic and anti-jamming performance;

4.Full function protection,;

5.Unit symbol display, very easy to reading;

6.Relative value, frequency/duty cycle measuring, data hold.

Technical

| Basic function | Range | Basic accuracy |
| --- | --- | --- |
| DCV | 400mV/4V/40V/400V/1000V | ±(0.5%+4) |
| ACV | 4V/40V/400V/1000V | ±(0.8%+6) |
| DCA | 400uA/4000uA/40mA/400mA/20A | ±(1.0%+5) |
| ACA | 400uA/4000uA/40mA/400mA/20A | ±(1.5%+5) |
| Resistance | 400Ω/4kΩ/40kΩ/400kΩ/4MΩ/40MΩ | ±(0.8%+1) |
| Capacitance | 4nF/40nF/ 400nF/4uF/40uF/200uF | ±(3.5%+8) |
| Frequency | 100Hz/1kHz/10kHz/100kHz/1MHz/30MHz | ±(0.5%+4) |
| Centigrade temperature | -40~1000℃ | ±(0.8%+4) |

| Special function | VC97 |
| --- | --- |
| Diode test | √ |
| hFE test | √ |
| Continuity test | Approx. less than 30Ω±10Ω |
| Low battery indication | Approx less than 2.4V |
| Duty cycle measuring | √ |
| Data hold | √ |
| Auto power off | Approx.15minutes |
| Function protect | √ |
| Input impendence | 10MΩ |
| Sampling rate | 3times/s |
| AC frequency response | 40-400Hz |
| Operation way | Manual or auto range |
| Max. display | 4000 |
| LCD size | 65x41mm |
| Battery | 3V AAA×2 pcs ( not included) |
| Power dissipation | Quiescent current approx. 2mA |

Package Content：

1x VC 97 Digital Multimeter

1x User Manual

2x basic cable