关键词：

Ublox NEO-M8N Flight Control GPS Accuracy for APM 2.6/ 2.8

Ublox NEO-M8N Flight Control GPS Support Beidou BDS Positioning Accuracy 2M for APM 2.6/ 2.8

Ublox NEO-M8N GPS Module Support for Ardupilot APM2.6 Flight Controller w compass

Ublox NEO-M8N GPS Module Compass with Shell & Folding Support for APM 2.6/ 2.8

Description:

- NEO-M8N has low consumption high precision, limiting precision 0.6m, most are about 0.9m according to tests

- Compared to NEO-7N 1.4-1.6m, the precision has been greatly improved, and support GPS/QZSS L1 C/A, GLONASS L10F, BeiDou B1

- Not including GPS holder

Specification:

* Receiver type 72-channel u-blox M8 engine

* GPS/QZSS L1 C/A, GLONASS L10F, BeiDou B1

* SBAS L1 C/A: WAAS, EGNOS, MSAS

* Galileo-ready E1B/C (NEO-M8N)

* Nav. update rate1 Single GNSS: up to 18 HZ

* Concurrent GNSS: up to 10 Hz

* Position accuracy2 2.0 m CEP

* Acquisition2 Cold starts: 26 s

* Aided starts: 2 s

* Reacquisition: 1.5 s

* Sensitivity2 Tracking & Nav:–167 dBm

* Cold starts:–48 dBm

* Hot starts:-156 dBm

* Assistance AssistNow GNSS Online

* AssistNow GNSS Offline (up to 35 days)3

* AssistNow Autonomous (up to 6 days)

* OMA SUPL & 3GPP compliant

* Oscillator TCXO (NEO-M8N/Q),

* Crystal (NEO-M8M)

* RTC crystal Built-In

* Noise figure On-chip LNA (NEO-M8M). Extra LNA for

* lowest noise figure (NEO-M8N/Q)

* Anti jamming Active CW detection and removal. Extra

*onboard SAW band pass filter (NEO-M8N/Q)

*Memory ROM (NEO-M8M/Q) or Flash (NEO-M8N)

*Supported antennas Active and passive

*Odometer Travelled distance

*Data-logger For position, velocity, and time (NEO-M8N)

*Operating temp. –40° C to 85° C

*Storage temp. –40° C to 85° C (NEO-M8N/Q)

* –40° C to 105° C (NEO-M8M)

*RoHS compliant (lead-free)

*Qualification according to ISO 16750

*Manufactured and fully tested in ISO/TS 16949 certified production sites

*Uses u-blox M8 chips qualified according to AEC-Q100

*Supply voltage 1.65 V to 3.6 V (NEO-M8M)

*2.7 V to 3.6 V (NEO-M8N/Q)

*Power consumption4 23 mA @ 3.0 V (continuous)

*5 mA @ 3.0 V Power Save Mode

*(1 Hz, GPS mode only)

*Backup Supply 1.4 to 3.6 V

*NEO-M8N-0 u-blox M8 Concurrent GNSS LCC Module,

*TCXO, flash, SAW, LNA,

*Timepulse Configurable 0.25 Hz to 10 MHz

Package included:

1 x NEO-M8N GPS Module

1x GPS stand